SPECIFICATION OUTLINE

Stratford Homes® are system-built modular homes.  Each home incorporates up-to date computerized technology, assembly line construction in a factory controlled environment, and a rigorous inspection process to ensure a quality built home and a superior finished product.

The material specifications for each assembly are listed below.   Within each assembly, the standard construction and finish items included in the base pricing structure for each home are listed first.  Following the standard material specifications is a partial listing of the many options available with every Stratford home.  We encourage you to consult with your Stratford Building Corporation representative if you have questions concerning any of the standard or optional construction and finish items available.

# FLOOR SYSTEMS

Floor Joist:	2x10” DFL #2 (or equiv) @ 16” o.c. for modules 14’ wide or less

2x12” DFL #2 (or equiv) @ 16” o.c. for modules wider than 14’

Rim Joist:		Double 2x10” or 2x12” DFL #2 to match floor joists

Floor Deck:		3/4” OSB T&G APA rated flooring, glued and ring-shank nailed

to floor joists

Optional items available: ¾” plywood subfloor, ½” AC plywood underlayment, and R-30 floor insulation with a poly vapor barrier

SIDEWALL SYSTEMS (EXTERIOR WALLS)

Wall Height:		8’

Wall Framing:		2x6” DFL (stud grade or equiv) @ 16” o.c.

Plates:			Single bottom and double top plate

Insulation:		R-21 fiberglass batts

Interior Wall:		1/2” sheetrock, taped and sanded, PVA sealer

Exterior Sheathing:	7/16” OSB APA approved sheathing

Builder’s Wrap: 		Primewrap .005” high-quality woven polyolefin infiltrating wrap

Optional items available: 9’ wall height, R-5 rigid insulation, ½” plywood wall sheathing

# INTERIOR WALLS

Wall Height:	8’

Wall Framing:	2x4” DFL studs @ 16” o.c. with single top and bottom plates; plumbing walls are 2x6” studs where noted on plans

Mateline Walls:	7/16” OSB exterior grade sheathing on mateline side of 2x4” studs

Optional items available: 9’ wall height, insulated interior walls, sound barrier on walls

1                                                           April 15, 2014

SIDING

Siding:	CertainTeed “MainStreet Double 4”  RigidForm 160 with “DuraLock” edges wood grain clapboard horizontal vinyl lap

Fascia:	Steel clad

Soffit:	CertainTeed fully vented enclosed vinyl soffit material (white)

Gable Trim:

Optional items available: Various vinyl siding profiles, cedar bevel siding, log-appearance

siding, cementitious siding products (HardiePlank or similar), cedar fascia , rough-sawn

fir plywood soffit, colored metal fascia, decorator accents

# ROOF/CEILING SYSTEM

Roof Load:		40 PSF

Trusses:		Engineered trusses at 24” o.c.; standard roof pitch is 4:12 on

ranch, split-entry, and 2-story models; 12:12 roof pitch standard

on Cape-Cod models; 10:12 on Tahoe model

Sheathing:		15/32” OSB APA rated sheathing, with “H” clips

Insulation:	R-50 Insulation (blown cellulose); R-30/R-38 fiberglass batts on Cape Cods and Tahoe model

Roofing:	Pabco “Premier®” Architectural Grade multi-layered laminate shingles, 30-year limited warranty, UL Class “A” Fire Rated, wind-resistant label over 15# felt, 24” “Ice Shield” material 24” above eave overhang

Venting:	Continuous Ridge Vent and perforated vinyl venting @ enclosed soffit

Gables and Eaves:	16” overhang

Optional items available: 6:12 and steeper roof pitches, Dutch hip and full hip roof designs, dormer accents, vaulted ceilings, tray ceilings, raised heel and energy heel trusses, 40 and 50-year shingles, higher roof loads to meet local design requirements, extended gable and eave overhangs, plywood roof sheathing

# WINDOWS

Windows:	Cascade Windows “WinPro Series” 9200 single hung with 3 1/4” frame width, ¾” Glass OA, Low “E” (.35 U-Value) double-glazing with screens, white frames; vented frames included at bedrooms

Window Finish:	Windows have sheetrock surround (matching walls) with wood sill and trim (finished to match door trim)

Optional items available: Bow and bay windows, window grids, argon gas fill, bronze

tinted glazing, skylights, shutters, almond and clay colored frames, wood windows

EXTERIOR DOORS

Exterior Doors:	Therma-Tru “Smooth Star” 6-panel fiberglass entry doors with solid polyurethane foam core; Size 30 at front entry, 28 at rear

Hardware:	Dexter “Solstice” series lever latch set; finish is satin chrome, bright chrome, brushed nickel or aged bronze

Patio Slider:	When shown on plans - 60610 Cascade Windows WinPro Model 9700 Vinyl Patio Door, with 4 3/8” frame width, 1” glass OA, Low “E” (.36 U-Value) with dual weather stripping at interlock and panels

Finish:	Primed

Optional items available: Optional door styles, sidelights, 20-minute rated fire doors,

painted or stained doors and sidelights, deadbolts and Schlage door hardware in various

finishes

2                                                          April 15, 2014

# PLUMBING

Supply Lines:	Cross-linked polyethylene (PEX) piping plumbed throughout house

DWV:			ABS plastic

Water Closet:	Kohler #K-3576 “Wellworth” round front floor mounted tank type

Color: White

Lavatory:	Kohler K-2196-4 “Pennington Series” 20” X 17 ½” Vitreous China lavatory.  Color: White

Bath Room Faucets:	Moen #6410 “Eva” series center set high arc faucet with sculptured lever handles, pop-up drain, lift rod, and one-piece ceramic valve; polished chrome finish

Tub/Showers:	Aquatic #2603 SGM- 60” bath and shower one-piece module, clear acrylic grab bar, molded-in floor texture, integral soap ledge.  Color: White

Tub/Shower Faucets:	Moen #T2133 “Eva” series bath and shower mixing valve with single lever polished chrome handle

Showers:	Aquatic #1363-C 36” or #1483 SG 48” one-piece acrylic fiberglass shower (white)

Shower Faucets:	Moen #T62132EP “Eva” series shower valve with single lever

polished chrome handle

Soaking Tub:	When shown on plans - Kohler K-1127 “Proflex” (60”x42” or Kohler #K-1145 “Proflex” (60”x60”) corner tub. Color: White w/synthetic marble surround

Kitchen Sink:	Kohler K-3346-4 “Toccata” stainless steel double basin sink

Kitchen Faucet:	Moen #7430 “Chateau” series single lever faucet with loop handle and matching side spray head

Bathroom Mirrors:	Plate glass, sized to match vanity

TP Dispenser:	Included (polished chrome)

Towel Bar:	One 24” bar at each bath (polished chrome)

Hose Bib:	Two each frost-free @ outside wall

Optional items available: Barrier free shower stalls, neo-angle showers, jets and in-line

heater for soaking tubs, brushed nickel or oil rubbed bronze bath faucets, glass tub and

shower enclosures, cast iron kitchen sinks, decorator faucets, pedestal sinks, laundry

tubs (freestanding or in cabinet), water heaters, grab bars

# ELECTRICAL

Service:	120V/240V Single Phase (underground or overhead service)

Service Panel:	200 amp Main Panel

Switches:	As required by layout (rooms are all individually switched);

Receptacles:	Residential grade duplex outlets throughout as required

Standard Circuits:	15-amp

Standard Wiring:	14 gauge Romex

GFI Outlets: 		Supplied as required by code

Exterior GFI Outlet:	Two each WP Outlets

Exhaust Fans:		50 cfm (vented to exterior) at bath rooms and utility rooms

Smoke Detectors:	Included and located as required by code

CO Detectors		Included and located as required by code

Optional items available: Light and heat/light bath fans, phone and television jacks,

speaker wires

3                                                             April 15, 2014

LIGHTING; LIGHTING FIXTURES

Interior Lights:	Ceiling mounted in bedrooms, walk-in closet, and hall

Kitchen:	One ceiling mounted plus one recessed can or down light @ sink

Dining Room:	5-Bulb chandelier profile fixture

Exterior Lights:		One fixture is included at each entry door, sliding glass door, and

patio swing door

Bathrooms:	Bar light over vanities, sized for vanity

Living Room:	No fixture; top half of wall receptacle switched

Optional items available: Various “Contemporary” style light fixture selections in chrome

and brushed nickel finishes, wall sconces, paddle fans (with or without lights), recessed

fluorescent lights, exterior lights with motion detector, track lighting, recessed can and

eyeball lights, under cabinet lighting, dimmer switches, Decora switches and recepts

INTERIOR FINISH

Interior Finish:	1/2” gypsum wallboard at walls and 5/8” at ceilings, taped ready for texture and paint

Vapor Barrier/Primer:	Sherwin Williams Moisture Vapor Barrier, Interior Latex

Color: White

Optional items available: Medium orange peel wall and ceiling texture, light knock down

ceiling texture, painted walls and ceilings (Sherwin Williams Pro-Mar interior Dover White

flat latex), 6-mil poly replacing PVA sealer, rounded drywall corners; stained 1x6”

T&G knotty alder

INTERIOR DOORS & TRIM

Interior Swing Doors:	Birch veneer stained hollow core doors w/wood jambs

Base Trim:	3 1/4” hemlock wood base trim, stained to match doors

Door Casing:	2 ¼” Colonial (profile # 356) hemlock, stained to match doors

Interior Door Hardware:	Dexter “Solstice”” series lever latch set in satin chrome, bright chrome, brushed nickel, or aged bronze; bathroom doors to have privacy locks

Bi-Fold Doors:	Hollow core birch veneer (stained to match)

Closet Jambs:	Sheetrock wrapped

Shelving:	Wire shelving at linen, pantry, and clothes closets

Optional items available: Bi-pass closet doors, stained oak, stained

alder, and white painted interior door and trim packages, Schlage door hardware,

chamfered and radius passageways, wood closet jambs and casing, wood wrapped

windows, spindle stair railing

KITCHEN, BATHROOM, UTILITY CABINETRY

Cabinets:	“Sundale” Essential Series maple front cabinets by Merillat

End Panels:		Photo finish overlay to match cabinets

Optional items available: Wide selection of cabinet styles, cabinet accessories, and finishes from Merillat Cabinets, soffits over kitchen cabinets, crown molding, beveled glass tri-view medicine cabinets

COUNTERTOPS

Kitchen and Baths:	Wilsonart square edge laminate top with 4” matching square edge laminate backsplash

Optional items available: 4’ tiled backsplash, bevel laminate edge, solid surface tops, synthetic marble formed top and vanity sink

4                                                           April 15, 2014

APPLIANCES

Range Hood:	2-Speed with light, ducted to exterior; Color: White

Optional items available: Plumb for icemaker, 1/2HP Badger garbage disposal, extensive

selection of Whirlpool appliances

HEATING AND AIR CONDITIONING

None included as standard feature

Optional items available: Gas flues installed, electric forced air wall heaters with

individual thermostats, baseboard hydronic heat systems, gas or wood-burning

fireplaces with hearths and mantels, prep for central forced air systems, radiant hot water

floor heating systems

FLOOR COVERINGS

None included as standard feature

Optional items available: Armstrong “Memories” vinyl flooring, Florida Tile ceramic floor

tiles (includes cement board sub flooring), Armstrong “Classics & Origins” laminate

wood flooring, Armstrong “Stones & Ceramics” stone appearance laminate flooring

5                                                                April 15, 2014